राष्ट्रीय  प्रौद्योगिकी  संस्थान  नागालैंड

NATIONAL INSTITUTE OF TECHNOLOGY NAGALAND

Chümoukedima, Nagaland -797103

TEXMiN - MINING CPS CENTRE OF EXCELLENCE

Project Guardian - Arduino Setup & Pin Configuration Manual

Welcome to the Arduino Hardware Guide for Project Guardian! This manual provides very detailed, easy-to-understand step-by-step instructions on how to use the Arduino codes

1. How to Upload the Arduino Code

Before wiring your sensors, you must upload the code to your ESP32 microcontrollers. Follow these steps for every Arduino code folder.

Step 1: Download and install the Arduino IDE on your computer.

Step 2: Install the ESP32 board manager in the Arduino IDE (Go to Tools -> Board -> Boards Manager and search for "ESP32").

Step 3: Install the necessary libraries: Go to Sketch -> Include Library -> Manage Libraries. Search for and install "LoRa", "Adafruit GFX", "Adafruit SSD1306", "DHT sensor library", and "MFRC522".

Step 4: Connect your ESP32 to your computer using a USB cable.

Step 5: Open the specific folder (e.g., WORKER) and double-click the .ino file (e.g., WORKER.ino).

Step 6: Select your ESP32 board and the correct COM Port under the Tools menu.

Step 7: Click the Upload button (the right-pointing arrow). Wait until it says "Done uploading".

2. Worker Node Configuration (WORKER.ino)

This node is carried by the worker. It has an OLED screen, an SOS button, and an Acknowledgment button.

Hardware Pin Connections

Always remember to connect VCC to power and GND to Ground on the ESP32.

OLED Display (I2C): SDA = Pin 21, SCL = Pin 22, VCC = 3.3V, GND = GND.

LoRa Module (SPI): SS/NSS = Pin 5, RST = Pin 14, DIO0 = Pin 2, SCK = Pin 18, MISO = Pin 19, MOSI = Pin 23, VCC = 3.3V, GND = GND.

Vibration Motor: Positive = Pin 25, Negative = GND.

Buzzer: Positive = Pin 26, Negative = GND.

SOS Button: One leg to Pin 27, the other leg to GND (Uses internal pullup).

ACK Button: One leg to Pin 33, the other leg to GND (Uses internal pullup).

3. Node 1 - Structural Health (node1vib.ino)

This node is installed at the entrance to monitor tunnel collapses and dangerous gases.

Hardware Pin Connections

MPU6050 (Accelerometer): SDA = Pin 25, SCL = Pin 26, VCC = 3.3V (or 5V depending on module), GND = GND.

SW420 Vibration Sensor: DO (Digital Out) = Pin 27, VCC = 3.3V or 5V, GND = GND.

DHT22 Temp & Humidity: Data Pin = Pin 21, VCC = 3.3V or 5V, GND = GND.

MQ7 Gas Sensor (CO): Analog Out (AO) = Pin 34, VCC = 5V, GND = GND.

MQ4 Gas Sensor (CH4): Analog Out (AO) = Pin 32, VCC = 5V, GND = GND.

LoRa Module (SPI): SS/NSS = Pin 5, RST = Pin 14, DIO0 = Pin 2, SCK = Pin 18, MISO = Pin 19, MOSI = Pin 23, VCC = 3.3V, GND = GND.

4. Node 2 - Environmental (node2withoutvib.ino)

This node is placed deeper inside the tunnel (Tunnel 1). It focuses purely on air quality and temperature.

Hardware Pin Connections

DHT22 Temp & Humidity: Data Pin = Pin 21, VCC = 3.3V or 5V, GND = GND.

MQ7 Gas Sensor (CO): Analog Out (AO) = Pin 34, VCC = 5V, GND = GND.

MQ4 Gas Sensor (CH4): Analog Out (AO) = Pin 32, VCC = 5V, GND = GND.

LoRa Module (SPI): SS/NSS = Pin 5, RST = Pin 14, DIO0 = Pin 2, SCK = Pin 18, MISO = Pin 19, MOSI = Pin 23, VCC = 3.3V, GND = GND.

5. RFID & Wi-Fi Gateway (rfidyolo.ino)

This node connects to your mobile hotspot via Wi-Fi and talks directly to the YOLO Python Server (Ngrok link) to verify worker access.

Hardware Pin Connections

RFID Reader 1 (Entry): SS/SDA = Pin 5, RST = Pin 22, SCK = Pin 18, MISO = Pin 19, MOSI = Pin 23, VCC = 3.3V, GND = GND.

RFID Reader 2 (Tunnel 1): SS/SDA = Pin 4, RST = Pin 15, SCK = Pin 18, MISO = Pin 19, MOSI = Pin 23, VCC = 3.3V, GND = GND.

Software Configuration

Before uploading this code, you must update the following lines in the "rfidyolo.ino" file:

Step 1: Change the Wi-Fi credentials to match your mobile hotspot: const char* ssid = "YourHotspotName"; const char* password = "YourPassword";

Step 2: Update the Flask Server URL with your YOLO Ngrok URL: const char* serverUrl = "https://your-ngrok-url.ngrok-free.dev/rfid";

6. Conclusion

Make sure to double-check all your VCC and GND connections before plugging in power. Incorrect voltage (like plugging a 3.3V LoRa module into 5V) can damage the components. Happy building!