TEXMiN - MINING CPS CENTRE OF EXCELLENCE

राष्ट्रीय  प्रौद्योगिकी  संस्थान  नागालैंड

NATIONAL INSTITUTE OF TECHNOLOGY NAGALAND

Chümoukedima, Nagaland -797103

Project Guardian - Laboratory & Modification Manual

Welcome to the Project Guardian manual! This guide is written in a simple, step-by-step manner so that anyone—can easily understand how to start, modify, and manage the system. Follow each step carefully like a recipe.

1. What is Project Guardian?

Project Guardian is a smart safety system designed to protect workers in dangerous places like mines or tunnels. It uses different technologies to work together:

• The Android App: A mobile application for the supervisor to monitor workers.

• The YOLO Camera System: A computer vision program that checks if workers are wearing safety helmets and vests.

• The Raspberry Pi Edge AI Server: A mini computer that collects data from sensors (like temperature and gas) and predicts if a tunnel might collapse.

• The LoRa Gateway: A wireless antenna that receives signals from deep inside the tunnel.

# System Requirements & Prerequisites

Before you begin running the commands, make sure you have all the necessary software and hardware installed and ready.

## Software Requirements

Python Version: Python 3.8 or higher is required on both the Windows computer and the Raspberry Pi.

NGROK: You must download and install Ngrok to create public links for the local servers.

LoRa Receiver Library: On the Raspberry Pi, you need to download the pySX127x library for LoRa communication to work properly.

Android Studio: Required to edit the Kotlin code and build the Android APK.

YOLO Environment: The Windows machine must have PyTorch, Ultralytics, and OpenCV installed to run the computer vision model.

## Hardware Requirements

Raspberry Pi: Acts as the Edge AI Server and LoRa Gateway.

Windows Computer: Acts as the YOLO PPE Server running the webcams.

LoRa SX127x Modules: One for the Raspberry Pi Gateway and one for the sensor nodes.

Sensors: Accelerometer, SW420 Vibration, DHT11 (Temperature/Humidity), MQ Sensors (CO/Methane).

Webcams: USB Webcams connected to the Windows computer for entrance and tunnel monitoring.

2. How to Run the System (Step-by-Step)

Here are the exact commands you need to type to bring the entire system online.

Part A: Running the Camera (YOLO PPE Detection)

This part runs on your main computer and uses the webcam to detect safety gear.

Step 1: Open your Command Prompt or Terminal on your computer.

Step 2: Navigate to the folder containing your YOLO model by typing exactly: cd Eagle-eyes-main\predictions\Model

Step 3: Run the YOLO python script. You can name the YOLO code "app.py" or use your own name. Type this command and press Enter: py -3.10 app.py

(Note: If you saved the "YOOLO MODEL Running .txt" code with a different name like "yolo_run.py", simply replace "app.py" with your filename).

Part B: Running the LoRa Receiver (On Raspberry Pi)

This script listens to the wireless antennas from the tunnel sensors. (Based on the "LORA CODE FOR PI.txt" file)

Important: While logging to the PI the password for the PI has been kept as kimi

Step 1: Turn on your Raspberry Pi and open its Terminal.

Step 2: If you saved your LoRa code as "receiver1.py" (or any other name), type this command to start it with administrative permissions: sudo python3 receiver1.

Part C: Running the Main AI Algorithm (On Raspberry Pi)

This script calculates risks and hosts the dashboard. (Based on the "Main Algorithm Code for AI in PI .txt" file)

Step 1: Open a second, new Terminal window on your Raspberry Pi.

Step 2: If you saved this code as "mine_monitor1.py", start the server by typing: sudo python3 mine_monitor1.py

Important: AI Model (.pkl file) Requirement

For the AI server to run successfully, the trained Machine Learning model file (e.g., building_health_model.pkl) MUST be present in the exact same folder as your Python script.

You have a few ways to get this file:

Download pre-trained models: You can train and download the .pkl file directly from platforms like Google Colab or Kaggle.

Train your own: You can create your own model using your custom dataset. You can name whatever file you want—just make sure to update the filename inside your Python code to match it!

Part D: Making the Server Public (Using Ngrok)

The Android App on your phone needs an internet link to connect to your Raspberry Pi.

Step 1: Open a third Terminal window on your Raspberry Pi.

Step 2: Start Ngrok by typing: ngrok http 5001

Step 3: Ngrok will give you a web link (URL) that looks like "https://xxxx-xxx.ngrok-free.app". Copy this link. You will need to paste this link inside your Android App code.

Part E: The Automated Alternative (PI Server Starter)

If you do not want to type all those Raspberry Pi commands one by one, we made an easy starter script (PI SERVER STARTER.txt).

Step 1: Run the starter code by typing: python3 starter.py (assuming you named the file starter.py).

Step 2: Open a web browser and go to http://localhost:9000/start-sensors. This simple click will automatically start the LoRa receiver, the AI server, and Ngrok all in the background for you!

Part F: Handling Mobile Hotspot & IP/Ngrok Changes

If your devices are connected to a Mobile Hotspot, their Local IP Addresses can change every time you connect. Ngrok URLs also change every time you restart Ngrok. Follow these steps when that happens:

How to find your new IP Address:

Step 1: On Windows: Open Command Prompt, type "ipconfig", and press Enter. Look for the "IPv4 Address" (e.g., 192.168.43.x).

Step 2: On Raspberry Pi: Open Terminal, type "hostname -I", and press Enter. The first number is your IP (e.g., 192.168.43.y).

Where to update the IP and Ngrok links in the Code:

Step 1: In the Android App ("MainActivity.kt"): Look at the very top of the file for "YOLO_SERVER" and "PI_SERVER". Replace the old Ngrok links with the new Ngrok URLs you generated in Part D.

Step 2: In the Raspberry Pi AI Server ("mine_monitor1.py"): If the PI needs to talk to the Windows YOLO server, look for the variable "YOLO_NGROK_URL = ..." and paste the new Windows Ngrok URL.

Step 3: In the Windows YOLO Server ("app.py"): Ensure it binds correctly by keeping `app.run(host="0.0.0.0", port=5000)`. It will automatically listen on your new Windows IP address.

Step 4: Direct IP connections (Without Ngrok): If you decide to connect directly without Ngrok, go to the Android App code and replace the "https://xxxx.ngrok.app" links with your direct IP addresses (e.g., "http://192.168.43.y:5001"). You must be on the exact same mobile hotspot for direct IP to work!

3. How to Make Modifications (For Developers)

If you want to edit or improve the project, here is how you can easily make changes.

Modifying the Android Application

Step 1: Open Android Studio and load the app.

Step 2: Open the "MainActivity.kt" file (which contains the "APP CODE.txt" contents).

Step 3: To change colors: Look for "Color.parseColor" in the code and change the #HEX code (for example, change #000000 to #FF0000 for red).

Modifying Safety Rules in the AI Server

Step 1: Open "mine_monitor1.py" in any text editor.

Step 2: Find the section near the top called "hazard_rules".

Step 3: If you want to make the system trigger a warning at a lower temperature, simply change the "threshold" number (e.g., change 40 to 35).

Modifying the YOLO Camera Model

Step 1: Open your YOLO script (e.g., app.py).

Step 2: Look for variables named "HELMET_LABELS" or "VEST_LABELS".

Step 3: If you train your camera to look for something new (like "goggles"), you can add it to this list.

4. Future Upgrades & Edge Computing Improvements

Since the primary goal of Project Guardian is Edge Computing (processing data locally instead of relying on external cloud servers), here are several ideas to make the system faster, more secure, and fully independent.

1. Moving Computer Vision to the Edge (Google Coral / Edge TPU)

Currently, the YOLO PPE detection requires a heavy Windows computer. By compressing the YOLO model using TensorRT or TensorFlow Lite, you can run the camera AI directly on the Raspberry Pi or an edge accelerator like the Google Coral USB Accelerator. This completely eliminates the need for the Windows PC.

2. Micro-Edge AI on the ESP32 (TinyML)

Right now, the ESP32 nodes stream raw sensor data to the Raspberry Pi over LoRa, and the Pi runs the ML models. Using TinyML (like TensorFlow Lite for Microcontrollers), you can put the anomaly detection directly onto the ESP32 chips. The ESP32 will only send a LoRa message when a hazard is detected, saving massive amounts of battery and wireless bandwidth.

3. Federated Edge Learning

Instead of retraining the ML models manually on a central computer, you can implement Federated Learning. Multiple Raspberry Pis in different tunnel zones can learn from new environmental anomalies locally. They can then share just their learned "weights" with each other to improve the global AI model without ever sending raw, sensitive data across the network.

5. Conclusion

By carefully following this manual, you can run the entire Project Guardian system from scratch. Take your time with each terminal command. Good luck with your project development!